GG-hexaWorks

GG-hexaWorks is an Excel Spreadsheet developed by IBRA members(Italian Bamboo Rodmakers Association)- Gabriele Gori from Florence (Italy) and Marco O. Giardina from Naples (Italy).

The spreadsheet is particularly useful for rodmaking above all with regards to the tapers.

It can be used to transform a Hexagonal taper into  Penta or Quad tapers.

The transformation is carried out in such a way to preserve the original MOI (Moment of Inertia) of the Hex rod. These transformations are often carried out by maintaining the same surface area, ie the Hex rod is transformed into  Penta or square rods having the same surface area.

By operating in this way you obtain rods having the same weight but that behave differently when flexed ie they have a different action.

TO make an example, it is common knowledge that rectangular strips of metal or wood will offer a different resistance to flexing whether they are flexed with either the wide side or the short side. The area is evidently the same but the behaviour under a flexing force is different.

Therefore the area is not the geometrical parameter which can describe the resistance to flexing, but instead it is the MOI (Moment of Inertia). In practice, maintaining the same material, the higher the MOI, the higher the resistance to flexing and the rod will be more rigid.

Furthermore, the further the material is from the centre of gravity of the section, the higher the MOI will be and it increases exponentially.

Getting back to the example of the rectangular strip, let’s suppose that the sides are respectively 2 and 4 cm. If the strip is placed flat (wide side), the Moment of Inertia of the section will be è B x H3 /12  = 4 x 23  /12 = 2,666 cm4

If instead it is placed along the short side it becomes:

B x H3 /12  = 2 x 43  /12 =  10,666 cm4

This means that the MOI is 4 times higher but the surface area is the same i.e.  A= 8 cm2.

Let’s leave the rectangle and move over to other geometrical figures, the formulas used to calculate the areas and the MOI will change but the concepts remain the same.

Therefore what counts is not only how much material is in the section – area – but in the evaluation of the behaviour to flexing what counts is how the material is distributed (MOI)

Another function of the spreadsheet is to facilitate the calculations and settings for the Morgan Hand Mill.

The use is simple: just choose the right sheet for the rod we are trying to make for example if we want to make a Quad from the taper of a Hex in three pieces, we will choose the sheet called MHM Quad 3 pcs; we will indicate that the original taper is Hexagonal by choosing “hex” in the cell and indicating the length of the rod in  inches.

In the cells below “Original Taper hex/quad” we will insert the original taper, taking care to insert the data from the Zero station to the one after the end of the Butt. For example if the rod is  96 inches long, insert the data until station 100, if it is 90 inches long – until station 95.

At this point in the next column we will read the taper which will have been transformed into a a quid with the same  MOI.

In the lower part of the spreadsheet, you will find the MHM settings for the three sections – tip, mid and butt for the 90° angle head as for the commonly used 92° and the values of the sizes of the strips in the various sections for both the  92° and  90° which is useful for the planning form.

The same work with a transformation of a Hex to a Penta. The spreadsheet also give indications for the sizes of ferrules. We believe that this work will be useful to rodmakers in a moment that rods with “non traditional” sections is having increasing success. This tendency is also due to the work by Tom Morgan and his brilliant Morgan Hand Mill which has in part revolutionized the  approach to Rodmaking with it incredible working flexibility in relation to this kind of taper.